2in1 Ski Mask/Beanie Measurements.

Because you are using a 100% Merino Wool Heavyweight Fluffy Knit, standard ski mask measurements will not work perfectly.

The "Fluff Factor": Fluffy knits have more volume and bulk than standard ribbed knits.

- Internal Circumference: The interior feels tighter than the exterior looks because the "fluff" takes up space.

- Folding Bulk: When you fold the bottom up to create the beanie mode, that fold will be very thick. You need extra length so the beanie doesn't end up looking too short or sitting too high on the head.

- Stretch: Heavyweight Merino has great stretch and recovery, but the fluffy texture means you don't want it too tight, or it will be uncomfortable on the face.

Here are optimum measurements designed for an Adult Unisex One-Size-Fits-Most (OSFM) fit, specifically tailored for a heavyweight, fluffy Merino knit.

These measurements should be taken with the garment laid flat, relaxed, and not stretched.

### Points of Measure (POM) Guide

| Code | Point of Measure Description | Optimum Measurement (Inches) | Optimum Measurement (CM) | Notes / Rationale for Fluffy Knit |
| --- | --- | --- | --- | --- |
| A | Total Length (Center Back) | 17.5" | 44.5 cm | Crucial: This is longer than a standard mask (usually 15-16"). The extra length is necessary so that when folded into a beanie, the bulky fluffy fabric doesn't make the hat too short. |
| B | Total Width (Across middle) | 9.5" | 24 cm | Slightly wider than average to account for the interior volume of the fluff. This equals a 19" circumference relaxed, stretching comfortably to 22-23". |
| C | Eye Opening Width (Horizontal) | 6.5" | 16.5 cm | A wide, clear opening is vital for comfort. The fluffy knit around the edges can sometimes impede vision if the hole is too small. |
| D | Eye Opening Height (Vertical center) | 3.0" | 7.5 cm | Tall enough to clear the eyebrows and the bridge of the nose comfortably. |
| E | Placement: Crown to Top of Eye | 7.25" | 18.5 cm | Measured from the very top center seam down to the top edge of the eye hole. This ensures the holes align with the average person's eyes. |
| F | Hem Opening Width (Bottom edge) | 9.0" | 23 cm | Slightly tapered in from the mid-width so it hugs the neck securely in ski mask mode. |
| G | Intended Cuff Fold Height | 3.5" | 9 cm | This is the recommended height of the fold when worn as a beanie. This height perfectly frames your 2"x1" brand tag. |

### Visualizing the Measurements

Here is how these measurements correspond to your design:

- In Ski Mask Mode: The 17.5" length will provide full neck coverage, tucking well into a jacket collar. The fluffy knit will feel warm and substantial around the face.

- In Beanie Mode: You will fold up the bottom 3.5" (Measurement G). Because the knit is fluffy, this fold will be thick and premium looking. The resulting beanie height will be roughly 10.5" (17.5 minus 3.5, plus a bit taken up by the bulk of the fold), providing a modern, slightly slouchy fit rather than a tight skullcap fit.

### Recommendation for your Manufacturer

When sending this to your manufacturer, add this note to the tech pack:

IMPORTANT MANUFACTURING NOTE:

Due to the "Heavyweight Fluffy Knit" construction, please ensure the interior circumference is checked against standard head forms. The measurements provided are exterior flat measurements. The bulk of the yarn should not compromise the interior fit. We require a pre-production sample to approve the stretch and recovery of this specific knit before bulk production.